
SECTION  14420
Supplement - 14420b


WHEELCHAIR LIFTS
Refer to the Construction Documents Directive for general editing procedures and for document coordination procedures.

UPDATE  -  29 Oct 1997

PART 1 GENERAL
 1.1
SUMMARY
A.
Includes But Not Limited To

1.
Furnish and install wheelchair lifts as described in Contract Documents.

B.
Related Sections

1.
Division 09  -  Carpeting

2.
Division 16  -  Conduit, wiring, boxes, and electrical power stub-in

 1.2
SYSTEM DESCRIPTION
A.
Design Requirements  -  Lift assembly shall provide stairs in raised position and function as a wheelchair lift when activated.

 1.3
SUBMITTALS
A.
Product Data  -  Manufacturer's literature, operating instructions, replacement parts lists, and maintenance data including description of operation and control, and schematic wiring diagrams.

B.
Shop Drawings  -  Provide accurate shop and erection drawings and diagrams, including required points of coordination with other trades to assure proper installation.

 1.4
QUALITY ASSURANCE
A.
Requirements of Regulatory Agencies

1.
Obtain and pay for necessary State and Municipal permits and perform such tests as may be required for acceptance and approval by Code authority.

2.
Work and material shall conform to requirements of National Electric Code and local codes which govern requirements of installation.

 1.5
WARRANTY
A.
Submit Manufacturer's 2 year unconditional warranty, including replacement labor, on motor.

 1.6
OWNER=S INSTRUCTIONS
A.
Instruct building maintenance personnel in proper operation, maintenance, and adjustment procedures utilizing Operation and Maintenance Manual.

PART 2 PRODUCTS
 2.1
MANUFACTURED UNITS
A.
Wheelchair Lift

1.
Capacity  -  750 pounds live load

2.
Speed  -  9.0 feet per minute minimum with full live load.

3.
Motor  -  3/4 hp, 1750 rpm, 110 volt, 60 cycle, single phase, 20 amp, instant reversing.

4.
Control Stations -

a.
3 sets of separate 24 volt pushbuttons for activating up and down operation of lift.  

b.
Pushbuttons momentary contact type requiring constant pressure for activation.

c.
Control station covers custom fabricated and finished to match adjacent surfaces.

5.
Construction  -  Steel framed assembly with wood stair risers and treads.  Assembly includes safety skirting around pit perimeter.

6.
Finish -

a.
All steel surfaces shall be prime coated.

b.
Surfaces exposed when lift is in lowered position shall be fabricated to readily receive carpeting.

7.
Approved Manufacturer -

Model number defines number of steps and height of rise.  Edit to fit project requirements.

a.
Step Lift Model LDS 2/18" C by Active Equipment Co, Sacramento, CA  (916) 649-1480.

PART 3 EXECUTION
 3.1
ADJUSTING
A.
Adjust equipment to assure smooth operation and accurate alignment at each stop level.


END OF SECTION

Project Number
1
14420

Project Name
Project Date
Wheelchair Lifts


