

SECTION  16140



WIRING DEVICES
Refer to the Construction Documents Directive for general editing procedures and for document coordination procedures.

See the Church Electrical Directives & Guidelines for additional information regarding the work of this Section.

UPDATE  -  9 Jul 1997

PART 1 GENERAL
 1.1
SUMMARY
A.
Includes But Not Limited To

1.
Furnish and install wiring devices complete with plates as described in Contract Documents.

Delete paragraph 2 if no work on sound system.

2.
Furnish and install cover plates for sound system devices.

B.
Related Sections

1.
Section 16055  -  General Electrical Requirements

PART 2 PRODUCTS
 2.1
GENERAL
A.
Faces shall be nylon where available.

B.
Devices of single type shall be from same Manufacturer.

Edit Paragraph C if other or more specific color requirements are necessary.

C.
Devices are listed as Ivory.  Use Ivory devices on light colored walls and brown on dark walls.

 2.2
MANUFACTURED UNITS
A.
Switches

The 'Designer' style devices specified in this optional paragraph 1 are intended for use on new Projects.

1.


Use this optional paragraph 1 specifying standard style devices where necessary on Addition/Remodel Projects to match existing device style. 

1.
Standard -

a.
Verified by UL to meet Fed Spec WS-896E.

Delete the items listed below in paragraph b which are not used on this Project.

b.
Approved Manufacturers & Models -

1)
20 amp, single pole

a)
Arrow Hart  -  1221I

b)
Eagle  -  2221V

c)
Hubbell  -  1221-I

d)
Pass & Seymour  -  20AC1-I

e)
Leviton  -  1221-2I

2)
Two Pole -

a)
Arrow Hart  -  1222I

b)
Eagle  -  2222V

c)
Hubbell  -  1222-I

d)
Pass & Seymour  -  20AC2-I

e)
Leviton  -  1222-2I

3)
Three Way -

a)
Arrow Hart  -  1223I

b)
Eagle  -  2223V

c)
Hubbell  -  1223-I

d)
Pass & Seymour  -  20AC3-I

e)
Leviton  -  1223-2I

4)
Four Way -

a)
Arrow hart  -  1224I

b)
Eagle  -  2224V

c)
Hubbell  -  1224-I

d)
Pass & Seymour  -  20AC4-I

e)
Leviton  -  1224-2I

5)
Pilot Switch -

a)
Arrow Hart  -  1991PL

b)
Eagle  -  2221-PL

c)
Hubbell  -  1221-PL

d)
Pass & Seymour  -  20AC1-RPL

e)
Leviton  -  1221-PLR

6)
Lighted Toggle Switch -

a)
Single Pole -

(1)
Arrow Hart  -  1991IL

(2)
Eagle  -  2221-LTV

(3)
Hubbell  -  1221-IL

(4)
Pass & Seymour  -  20AC1-ISL

(5)
Leviton  -  1221-LHI

b)
Three Way -

(1)
Arrow Hart  -  1993IL

(2)
Eagle  -  2223-LTV

(3)
Hubbell  -  1223-IL

(4)
Pass & Seymour  -  20AC3-ISL

(5)
Leviton  -  1223-LHC

Edit to fit Project circumstances.

B.
Receptacles

The 'Designer' style devices specified in this optional paragraph 1 are intended for use on new Projects and additions to recent phase buildings to match existing devices.

Use standard style devices specified in this optional paragraph 1 where necessary on Addition/Remodel Projects to match existing device style. 

1.
Standard Style -

a.
15 amp, specification grade, back and side wired, self grounding.

b.
Verified by UL to meet Fed Spec WC-596F.

c.
Approved Manufacturers & Models -

1)
Arrow Hart  -  5252I

2)
Eagle  -  5252V

3)
Hubbell  -  5252-I

4)
Leviton  -  5252-I

5)
Pass & Seymour  -  5252-I

2.
Ground Fault Circuit Interrupter (GFCI) -

a.
15 amp, specification grade.

b.
Approved Manufacturers & Models -

1)
Arrow Hart  -  GF5242I

2)
Eagle  -  646-2V

3)
Hubbell  -  GF5252I

4)
Leviton  -  6599-I

5)
Pass & Seymour  -  1591-I

C.
Telephone Jacks

1.
Wall type, 4 conductor, screw terminals.

a.
Approved Manufacturers & Models -

1)
Eagle  -  TWD3521V

2)
Leviton  -  40257-I

3)
Pass & Seymour  -  WMTE14-I

4)
Suttle  -  SE-630AC4-50

D.
Plates

1.
Standard Cover Plates -

a.
Nylon or high impact resistant thermoplastic.

b.
Color shall match wiring device.

c.
Ganged switches shall have gang plates.

d.
Approved Manufacturers -

1)
Arrow hart

2)
Eagle

3)
Hubbell

4)
Leviton

5)
Pass & Seymour

PART 3 EXECUTION
 3.1
INSTALLATION
A.
Install devices flush with walls, straight, and solid to box.

B.
Label dimmer switch groupings with 1/16 inch thick laminated plastic composition material with contrasting color core.  Engraved letter shall be 1/4 inch high.


END OF SECTION

ADVANCE \d252


RENOVATE BOOKSTORE/
16140-1
Wiring Devices 

STUDENT SERVICES AREA


SUNY College at Old Westbury

SUCF Project No. 31221-808P1


